V5.1/V5.2 Protocols

# Off-Line Analysis on PC

The ISDN PRA Tester (IBT-200) and ISDN BRA/PRA Tester (IBT-300) offer a built-in full dedoding of V5.1 and V5.2 frames on up to 3 time-slots simultaneously, from frame summary to frame detail, down to hexadecimal values.

You have also requested the possibility to analyse V5.1/V5.2 monitoring files off-line on PC, for ease of use and reporting purposes.

This feature is not available in ISDNpartner, but you may use the Windows(TM) PC Detailed Decoder (V6.01) to achieve this.

Thanks to this small conversion program, you are able translate any *.mon file form IBT-200/-300 to *.ibt format. In addition, the converter will create one *.ibt file for each time-slot monitored by the IBT-200/-300.

These converted files can then be displayed in the Windows(TM) PC Detailed Decoder (V6.01), which offers the same level of detail as in the tester itself, as well as  powerful filters.

Instructions:

- Place the file in an empty directory, together with the V5.x monitoring file you generated with IBT-200/-300.

- At DOS prompt, type "mon2ibt" and the name of your .mon file.

- The tool will convert the .mon  file to .ibt format.

- You may open this .ibt file in our Windows(TM) PC Detailed Decoder software (V6.01), which is delivered with every IBT-10 and IBT-20. This software decodes V5.x frames (whereas ISDNpartner doesn't). If you don't have it, I can send it to you (3.1 MB).

Note: As you may monitor up to 3 time slots with IBT-200/-300, the tool will create one .ibt file for every time slot monitored.